Protocol Map and port

Must know:

Ethernet — Layer 2 framing.

ARP — IP → MAC.

IP — logical addressing and routing.

ICMP — ping, traceroute, errors.

TCP — connection-oriented, reliable, 3-way handshake.

UDP — connectionless, fast, no guarantee.

DNS — names → IPs.

DHCP — automatic IP config.

HTTP/HTTPS — web.

TLS — encryption, integrity, authentication for HTTPS.

QUIC — UDP-based transport for HTTP/3.

TCP 3-way handshake: SYN → SYN-ACK → ACK.

TCP teardown: FIN/ACK → ACK → FIN/ACK → ACK, or RST.

Ports to memorize:

22 SSH

53 DNS

67/68 DHCP

80 HTTP

123 NTP

443 HTTPS / QUIC

3389 RD

Also useful: 25 SMTP, 110 POP3, 143 IMAP, 161 SNMP, 389 LDAP, 445 SMB, 3306 MySQL, 5432 PostgreSQL.

Sequence: DNS → ARP → TCP → TLS → HTTP.

- DNS: the browser looks up for the IP address from the website, example.com,

- ARP: if the destination is in local address or needs to find the physical addres of the gateway/router. Translates that IP address to a MAC address

- TCP: a safe and secured connection opens up to the server through the 3-way handshake

- TLS: TLS handshake happens, an encrypted conection gets established where there is an exchange of secret security codes/keys so no one can read or steal the information that is getting exchanged in that connection

- HTTP: the information from the app goes through a safe, and established chanel and the server replies with the website data where from the user’s device it can be seen the webpage.

Block 4 – Hands-on capture

ip a

- Will show the device’s network interface configuration. In my laptop has two configs: 1. lo: <LOOPBACK,UP,LOWER_UP> and  2. eth0: <BROADCAST,MULTICAST,UP,LOWER_UP>

ip route

- Is showing to the admin/ what packets are being sent from the computer to the dest

arp -a

- It shows the ARP cache: IPv4 → MAC mappings your host currently knows. “My address name” isn’t right — it’s the IP-to-MAC table, usually the gateway and any recently contacted local hosts. Currently in my local linux only shows one which is my address name, HWtype, HWaddress, Flags Mask and Iface.

dig example.com

- Is interrogating DNS name servers and showing a DNS lookup. For the url example.com it will show us the following: if there was an error in the lookup, the ip address from where the server is, the port and the date and message size of the lookup.

traceroute example.com

- This tool helps us to track the route packets to the network host. For example.com, there were a total of 7 hops telling us how in each step/hop where our packets/traffic was going. This will help me understand better if there is any failure in the packets sent to a certain website or something along those lines

ss -tulnp

- Is a tool to investiaget sockets. Similar to netstat, it displays information about tcp and other state information. When using the -tulnp it will display all active TCP and UDP listening ports with their specific process name, process ID and their local addres

curl -v https://example.com

- This tool is for transferring data from or to a server using URLs, it supports multiple protocols such as DICt, FIL, FTP/S, GOPHER/S, HTTP/S… It is showing a list of the host, IPv4 and v6, connected to which ipv4 address and port, show if the 3-way handshake was successful or not, the SSL connection and more much info about the data that got transferred to the url.

Wireshark packets
I got 90 frames recorded of my own device network and I’ll be answering the following questions to see if I actually managed to understand wireshark.

What IP did DNS return for example.com? 172.66.147.243 and 104.20.23.154

What’s the source port of your TCP SYN to 443? (ephemeral port) 43702

What TLS version did the Client Hello offer? What version did the server pick? Client: TLS 1.2 | Server: TLSv1.3

What SNI was sent in the Client Hello? example.com

What was the destination MAC in the SYN — the gateway or the server? Microsoft_8f:c1:c8 (00:15:5d:8f:c1:c8) - it is the gateway

Did you see any ARP packets? What did they ask? Who has <gateway IP>? Tell <your IP>” — the reply is “<gateway IP> is at <gateway MAC>”

How many TCP handshake packets start the connection (SYN, SYN-ACK, ACK)? 3 packets

Was there any plaintext HTTP in the capture? Why or why not? Yes there was some HTTP in the capture in the 64 and 67 frames. There was some because in the linux machine, curl -v http://example.com was used to capture some data and see the difference from other protocols.

Block 5:
1. Type https://example.com

2. DNS query → UDP/53 → response: A record = <IP>

3. ARP: who has <gateway>? reply → <gateway MAC>

4. TCP SYN → <server IP>:443 (src port <ephemeral>)

5. TCP SYN-ACK ← server

6. TCP ACK → server

7. TLS ClientHello → SNI=example.com, versions=<...>

8. TLS ServerHello → chosen version=<...>, cipher=<...>

9. Certificate exchange → validate chain

10. TLS Finished both sides

11. HTTP GET / (encrypted)

12. HTTP 200 OK (encrypted)

13. Assets fetched, page renders

14. TCP FIN/ACK teardown (or reuse / keep-alive)

Extra information:

- DNS is unencrypted by default  spoofable  mitigations: DNSSEC, DoH, DoT

- ARP is unauthenticated  spoofable  MITM   mitigations: Dynamic ARP Inspection, static ARP, 802.1X

- TLS protects payload if certs validate  otherwise MITM possible

- HTTP plaintext = readable on wire  use HTTPS + HSTS